Software Specification

Fibre Infrastructure Mapping and Traceability System

# 1. Document Purpose

This document defines the proposed MVP software specification for the Fibre Infrastructure Mapping and Traceability System. It is intended for stakeholder review and sets out the scope, goals, functional expectations, and delivery boundaries for the first version of the system.

The application is intended to model physical fibre infrastructure visually and logically, allowing users to map locations, install equipment, define fibre connectivity, and trace fibre paths from a selected port to its destination.

# 2. Technology Stack

Blazor

C#

SQL Server

# 3. Purpose and Primary Goal

The system is intended to provide a web-based application that allows users to upload and manage site maps, place physical areas on those maps, create racks within those areas, install fibre-related components from templates, patch fibre connections between ports, track fibre usage core-by-core, and trace complete or partial fibre paths from a selected port through to its destination.

The primary goal for the MVP is to provide a working system where a user can upload a site map, place rooms and equipment containers on that map, add racks within those areas, populate racks with templated components, define fibre cables and fibre-core relationships, patch connections between relevant ports, and click a port to see the resulting path trace both visually and as a text/list output.

# 4. Intended Users

Engineers

Technicians

Planners

Maintainers

Operations personnel

# 5. MVP Scope

## 5.1 In Scope

Create and manage sites.

Upload site maps in PNG or JPG format.

Zoom and pan around the site map.

Place draggable and resizable boxes over the site map.

Support area types of Room and Equipment Container.

Treat an Equipment Container as a physical enclosure or mini-location, such as a conveyor starter.

Create racks inside rooms or equipment containers.

Display racks visually so installed components and ports can be interacted with.

Provide a template system and template designer.

Support templated sub-components such as switches, PLCs, patch panels, FOBOTs / fibre terminations, and other fibre-related devices.

Support fibre only.

Support singlemode and multimode fibre.

Track fibre usage core-by-core.

Support duplex pairs.

Allow patching and connectivity between relevant ports and terminations.

Allow partial traces.

Show both a visual path trace and a text/list trace when a user clicks a port.

## 5.2 Out of Scope

Copper cabling

Wall outlets

Work orders / moves-adds-changes workflow

Enterprise authentication integrations such as SAML, LDAP, or MFA

GIS integration

Mobile-first field application

Advanced dashboards and reporting

Change-management workflow engine

Automated naming enforcement

Advanced icon designer or shaped overlays

# 6. Conceptual Hierarchy

Site

Map

Areas

Room

Equipment Container

Racks

Sub-components

Ports

Connections

Fibre cores

Trace paths

An Equipment Container is not the same as a switch or PLC. It behaves more like a physical enclosure or mini-location that can contain racks and sub-components.

# 7. Functional Expectations

## 7.1 Sites and Maps

Create a site.

Upload and update a site map.

Zoom and pan around the map.

## 7.2 Areas

Add room and equipment container boxes on the map.

Drag and resize area boxes.

Capture area metadata including name, type, and optional description.

## 7.3 Racks

Add racks inside an area.

Maintain rack name, rack size / unit count, and notes.

Provide a visual rack representation suitable for interaction.

## 7.4 Components and Templates

Create and manage component templates.

Place templated components into racks.

Allow direct placement into an area for non-rack-mounted items where needed.

Ensure ports are visually clickable.

## 7.5 Fibre and Connections

Define fibre cables with attributes such as name, type, core count, origin, and destination.

Represent fibre cores individually.

Model duplex relationships.

Patch ports together using fibre/core relationships.

## 7.6 Trace

Click a switch port or other component port.

Show the resulting path both visually and in text/list form.

Identify fibre cores used, duplex pair used, intermediate terminations or hops, end destination, and any incomplete segment if the path is partial.

# 8. User Interface / Screens

Site Map Screen

Area Detail Screen

Rack View Screen

Template Designer Screen

Connection Management Screen

Trace Screen / Panel

# 9. Naming Approach

A naming approach will be needed in the future, but the MVP should not hard-code a fixed naming standard. The MVP should allow manual naming, support structured fields, and avoid enforcing rigid naming rules at this stage.

# 10. Technical Direction and Design Principles

Maintainable internal application, not a throwaway prototype

Correctness of fibre, core, and path modelling

Clean and practical Blazor UI

Simple but expandable data model

Visible and usable rack interaction

Reliable traceability from a clicked port to destination

Avoid React / TypeScript

Avoid unnecessary over-engineering

# 11. Acceptance Criteria

Create a site.

Upload a PNG or JPG map.

Place rooms and equipment containers on the map.

Create racks in those areas.

Create component templates.

Place templated components into racks.

Define fibre cables and fibre cores.

Patch ports together using fibre/core relationships.

Click a switch port or other component port.

See both a visual path and a text path.

Identify the fibre cores and duplex pair used.

See where a trace is incomplete when only a partial path exists.

# 12. Suggested Delivery Phases

## Phase 1 — MVP

Sites

Maps

Areas

Racks

Template designer

Installed components

Ports

Fibre cables and cores

Basic patching

Basic trace output

## Phase 2 — Future Enhancements

Better visuals and icons

Richer trace graphics

Naming rules

Reports

Dashboards

Advanced filtering

Bulk editing

Operational workflows

# 13. Summary

This specification defines an MVP focused on one clear outcome: a practical system for visually mapping fibre infrastructure and tracing fibre connectivity accurately from port to destination. The application should be built with enough structure to be maintainable and expandable, while keeping the initial scope disciplined.